S3 SUMMARY (November 2024)

Cherokee County

Highway 3 over Little Sioux River

NHSX-003-2(81)—3H-18

Project Extent

The overall project consists of roadway improvements, a bridge replacement, and new overflow bridge along Highway 3 over the Little Sioux River near Cherokee, Iowa. The project starts at Station 945+00 and ends at Station 972+00, a distance of about 2700 feet. The final configuration of the roadway will consist of two, 12-foot lanes in each direction with 10-foot shoulder widths. The roadway alignment will remain the same, with minor grade changes to the approach embankments.

Index of Sheets, Tabs, and Documents

Q.1--Q.5 – Soil Legend and Symbol Information

Soil Meshes – Cross Sections

CS.1 – Tab Sheets

103-6 – Embankment with Moisture Control

103-7 – Shrinkage Data

104-9 – Longitudinal Subdrain Shoulder and Backslope

S3 Report

Van Dyke Report

PDFs of the Q and CS Sheets, the S3 report, the Van Dyke report, and soil mesh.dgn files are located: PW:\Projects\1800303020\Soils\S3 Submittal\(81)\.  A link to this directory is provided: (81)

Bridge, Culverts, and Interchanges

Two separate S4 reports (82 and 83) were submitted to Iowa DOT for the Highway 3 replacement bridge over the Little Sioux River (82) and the overflow bridge (83).  They can be found at the following links:

Replacement Bridge:  (82)

Overflow Bridge:  (83)

Soil Meshes

Soil meshes are complete. They can be found at the following link: (81)

Shrinkage

Class 10 30%; Topsoil 40%; and Boulder Estimate 10 Cu. Yds.

Moisture Control

Moisture Control is required on all Class 10 fill (except topsoil). See Tab 103-6.

Topsoil Removal

Topsoil Replacement Policy applies. See soil meshes.

Longitudinal Subdrain Tab

A Longitudinal Subdrain Tab is included with this grading project. See the CS sheets, and Tab 104-9 for details. Special attention should be paid to all detail notes on the tab.

List of Bid Items and Quantities

| Bid Item | Bid Item Number | Quantity | Tab Number |
| --- | --- | --- | --- |
| Boulder Estimate | 2102-2712015 | 10 Cu. Yds. | 103-7 |
| 4” Longitudinal Subdrain Pipe (perforated) | 2502-8212204 | 4230 ft. | 104-9 |
| Longitudinal Subdrain Outlets, DR-306 | 2502-8221306 | 24 outlets | 104-9 |

Other-Please Note:

Moisture Control.  Designer to calculate quantity.  See Tab 103-6.

Shrinkage Data.  Designer to apply shrink factors listed above.

Topsoil Stripping and Replacement.  Designer to calculate quantity.  See soil meshes.